Technical Recommendation

Model：BRTIRUS0707A

Name：Six axis robot

Robot version: 21 version

（Without further notice if specification and exterior have been changed because of improvement. Thanks for your understanding.）

# 1. BRTIRUS0707A robot profile

  - 1.1.Introduction of BRTIRUS0707A

BRTIRUS0707A is a six axis robot developed by BORUNTE for certain monotonous, frequent, and repetitive long-term operations, or in dangerous and harsh environments. It has a 883mm arm span and a maximum load of up to 7kg. With multiple degrees of flexibility, it can be used in polishing, assembling, and spraying.

High IP code：IP65  Dust-proof and water-proof

High accuracy：repeated positioning accuracy: ±0.03mm

## 1.2. Application cases of BRTIRUS0707A

(1). assembly (2). spraying (3). loading and unloading, etc.

# 2. BRTIRUS0707A robot specifications

| Model |  | BRTIRUS0707A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Axes |  | 6 axis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Loading ability |  | 7kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IP code |  | IP65 Dust-proof and water-proof |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Repeated positioning accuracy |  | ±0.03mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum arm span |  | 883mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight |  | 55kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max speed |  | J1 |  |  |  | 220.8°/s |  |  | J2 |  |  |  |  |  | 270°/s |  |
|  |  | J3 |  |  |  | 375°/s |  |  | J4 |  |  |  |  |  | 308°/s |  |
|  |  | J5 |  |  |  | 300°/s |  |  | J6 |  |  |  |  |  | 342°/s |  |
| Motion range |  | J1 |  |  |  | ±174° |  |  | J2 |  |  |  |  |  | -125°/+85° |  |
|  |  | J3 |  |  |  | -60°/+165° |  |  | J4 |  |  |  |  |  | ±180° |  |
|  |  | J5 |  |  |  | ±120° |  |  | J6 |  |  |  |  |  | ±360° |  |
| Allowable torque |  | J4 |  | 20.8N*m |  |  |  | J5 | 33.6N*m |  |  | J6 |  |  |  | 16.83N*m |
| Allowable moment of inertia |  | J4 |  | 0.17kg*m2 |  |  |  | J5 | 0.62kg*m2 |  |  | J6 |  |  |  | 0.155kg*m2 |
| Mounting environment |  | Operating temperature： 0℃~40℃ Best temperature：15℃~25℃ Relative humidity：20~80%RH（No condensation） |  |  |  |  |  |  |  |  | Mpa |  | 0.5~0.7MPa |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Power supply |  | 220V±10％ 50Hz/60Hz |  |  |  |
| Core components |  | Name |  |  | Operating system |  |  |  |  | Brand |  |  |  | BORUNTE |  |  |
|  |  |  |  |  | Casting body |  |  |  |  |  |  |  |  | BORUNTE |  |  |
|  |  |  |  |  | Servo motor |  |  |  |  |  |  |  |  | BORUNTE |  |  |
|  |  |  |  |  | Reducer |  |  |  |  |  |  |  |  | BORUNTE |  |  |
|  |  |  |  |  | Electrical component |  |  |  |  |  |  |  |  | BORUNTE |  |  |
| Oil content |  | J4 |  |  |  |  | 50ML |  |  |  |  |  |  |  |  |  |
| Vulnerable parts | J6timing belt | MBT233060306 Timing belt HP-3M-06 Bandwidth - 145 Toothed belt length 435 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | J5timing belt | MBT233060307 Timing belt HP-3M-06 Bandwidth - 100 Toothed belt length 300 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | J3timing belt | MBT233090508 Timing belt HP-5M-09 bandwidth - 95 tooth belt length 475 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | J1timing belt | MBT233090506 Timing belt HTD-5M-HP-68 teeth - 9mm wide |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Robot accompany materials |  |  | USB |  |  |  |  |  |  |  |  |  |  |  |  |  |

The rated power, specifications, external dimensions, etc. of this product are subject to modification without prior notice. Technical data and illustrations are for supply reference only and reserve the right to make changes.