Technical Recommendation

Model：BRTIRUS2110A

Name：Six axis robot

Robot version :21 version

（Without further notice if specification and exterior have been changed because of improvement. Thanks for your understanding.）”

# 1. BRTIRUS2110A robot profile

  - 1.1.Introduction of BRTIRUS2110A

BRTIRUS2110A is a six axis robot developed by BORUNTE for complex applications with multiple degrees of freedom. It has a 2131mm long arm span and a maximum load of 10kg. With six degrees of flexibility, it can handle scenarios such as welding, loading and unloading, and assembling.

High accuracy：repeated positioning accuracy: ±0.05mm

## 1.3. Application cases of BRTIRUS2110A

(1). welding (2). injection molding (3). stamping (4). loading and unloading, etc.

# 2.BRTIRUS2110A robot specifications

| Model |  | BRTIRUS2110A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Axes |  | 6 axis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Loading ability |  | 10kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Repeated positioning accuracy |  | ±0.05mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum arm span |  | 2131mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight |  | 230kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max speed | J1 | 110°/s |  |  | J2 |  |  |  |  | 146°/s |  |  |  |  |  |  |
|  | J3 | 134°/s |  |  | J4 |  |  |  |  | 273°/s |  |  |  |  |  |  |
|  | J5 | 300°/s |  |  | J6 |  |  |  |  | 342°/s |  |  |  |  |  |  |
| Motion range | J1 | ±155° |  |  | J2 |  |  |  |  | -140°/+65° |  |  |  |  |  |  |
|  | J3 | -75°/+110° |  |  | J4 |  |  |  |  | ±180° |  |  |  |  |  |  |
|  | J5 | ±115° |  |  | J6 |  |  |  |  | ±360° |  |  |  |  |  |  |
| Allowable torque | J4 | 52N*m |  | J5 |  | 33.6N*m |  |  |  |  |  | J6 |  |  | 16.8N*m |  |
| Allowable moment of inertia | J4 | 0.664kg*m2 |  | J5 |  | 1.12kg*m2 |  |  |  |  |  | J6 |  |  | 0.155kg*m2 |  |
| Mounting environment | Operating temperature： 0℃~40℃ Best temperature：15℃~25℃ Relative humidity：20~80%RH（No condensation） |  |  |  |  |  |  | Mpa |  |  |  |  |  | 0.5~0.7MPa |  |  |
|  |  |  |  |  |  |  |  | Power supply |  |  |  |  |  | 220V±10％ 50Hz/60Hz |  |  |
| Core component | Name | Operating system |  |  |  |  |  |  | Brand |  |  |  | BORUNTE |  |  |  |
|  |  | Casting body |  |  |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |
|  |  | Servo motor |  |  |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |
|  |  | Reducer |  |  |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |
|  |  | Electrical component |  |  |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |
| Oil content | J1 | 1400ML | J2 |  |  |  | 1000ML |  |  |  | J3 |  |  |  |  | 800ML |
|  | J4 | 250ML | J5 |  |  |  | / |  |  |  | J6 |  |  |  |  | / |
| Vulnerable parts | Timing belt | Tooth shape 3M, 100 teeth, 6mm wide, reinforced |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Timing belt | Tooth shape 3M, 145 teeth, 6mm wide, reinforced |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Robot accompanying materials |  | USB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The rated power, specifications, external dimensions, etc. of this product are subject to modification without prior notice.

Technical data and illustrations are for supply reference only and reserve the right to make changes.