Technical Recommendation

Model: BRTYZGT02S2B

Name：Two axis robot

Robot version :21 version

（Without further notice if specification and exterior have been changed because of                                   improvement. Thanks for your understanding.）

- BRTYZGT02S2B robot profile

Introduction of BRTYZGT02S2B

  - The BRTYZGT02S2B is a die casting ladling machine developed for automatic molten metal ladling applications. It is suitable for die casting machines from 160T to 400T and has a maximum ladle loading capacity of 4.5 kg. The machine adopts an integrated drive control system with fewer signal lines, making maintenance simpler and operation more stable. It is equipped with a convenient handheld operating device, allowing clear parameter and function settings. The complete structure is driven by a servo motor and RV reducer, providing stable, accurate, and efficient operation during the ladling process. With a cycle time of approximately 6.23 seconds, the BRTYZGT02S2B is suitable for efficient and reliable die casting production environments.

## 1.2. Application cases of BRTYZGT02S2B

(1). Die casting ladling
(2). Molten metal pouring
(3). Aluminum alloy die casting
(4). Automatic feeding for die casting machines
(5). Foundry production automation

# 3. BRTYZGT02S2B robot specifications

| Items | BRTYZGT02S2B |
| --- | --- |
| Applicable to die casting machine | 160T–400T |
| Manipulator Motor Drive (kW) | 1 kW |
| Table Spoon Motor Drive (kW) | 0.75 kW |
| Arm reduction ratio | RV40E 1:153 |
| Ladle reduction ratio | RV20E 1:121 |
| Max. loading (kg) | 4.5 kg |
| Recommended table spoon type | 0.8 kg–4.5 kg |
| Table Spoon Max. (mm) | 350 mm |
| Recommended height for smelter (mm) | ≤1100 mm |
| Recommended height for smelter arm | ≤450 mm |
| Cycle Time | 6.23 s within 4 s, the arm standby position starts to descend until the soup is injected |
| Main control power | AC Single phase AC220V / 50Hz |
| Power Source (kVA) | 0.93 kVA |
| Dimension | Length × width × height: 1140 × 680 × 1490 mm |
| Weight (kg) | 220 kg |

The rated power, specifications, external dimensions, etc. of this product are subject to modification without prior notice.

Technical data and illustrations are for supply reference only and reserve the right to make changes.