Technical Recommendation

Model：BRTIRUS1510A

Name：Six axis robot

Robot version :21 version

（Without further notice if specification and exterior have been changed because of improvement. Thanks for your understanding.）

- BRTIRUS1510A robot profile

## 1.1. Introduction of BRTIRUS1510A

BRTIRUS1510A is a six axis robot developed by BORUNTE for complex applications with multiple degrees of freedom, with a maximum load of 10kg and a 1587mm arm span. Adopting a lightweight arm design, the mechanical structure is compact and concise, and can perform flexible operations in a narrow workspace under high-speed motion. With six degrees of flexibility, it can freely handle scenarios such as loading and unloading, injection molding, die-casting, assembling, gluing, polishing, and testing. Easy to operate - use Huacheng control system. Suitable for injection molding machine: 200T-600T.

High IP code：IP54

High accuracy：repeated positioning accuracy: ±0.05mm

## 1.2. Application of BRTIRUS1510A

(1). welding (2). injection molding (3). stamping (4). loading and unloading, etc.

- BRTIRUS1510A robot specifications

| Model |  |  | BRTIRUS1510A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Axes |  |  | 6 axis |  |  |  |  |  |  |  |  |  |
| Loading ability |  |  | 10kg |  |  |  |  |  |  |  |  |  |
| IP code |  |  | IP54 |  |  |  |  |  |  |  |  |  |
| Repeated positioning accuracy |  |  | ±0.05mm |  |  |  |  |  |  |  |  |  |
| Maximum arm span |  |  | 1587mm |  |  |  |  |  |  |  |  |  |
| Weight |  |  | 152kg |  |  |  |  |  |  |  |  |  |
| Max speed | J1 |  | 190°/s |  |  | J2 |  |  | 173°/s |  |  |  |
|  | J3 |  | 223°/s |  |  | J4 |  |  | 254°/s |  |  |  |
|  | J5 |  | 270°/s |  |  | J6 |  |  | 342°/s |  |  |  |
| Motion range | J1 |  | ±165° |  |  | J2 |  |  | -95°/+70° |  |  |  |
|  | J3 |  | -85°/+75° |  |  | J4 |  |  | ±180° |  |  |  |
|  | J5 |  | ±115 |  |  | J6 |  |  | ±360° |  |  |  |
| Allowable torque | J4 | 32.8N*m |  | J5 | 32.8N*m |  |  |  | J6 | 17N*m |  |  |
| Allowable moment of inertia | J4 | 0.46kg*m2 |  | J5 | 0.62kg*m2 |  |  |  | J6 | 0.155kg*m2 |  |  |
| Mounting environment | Operating temperature： 0℃~40℃ Best temperature：15℃~25℃ Relative humidity：20~80% RH（No condensation） |  |  |  |  |  |  | Mpa |  |  | 0.5~0.7MPa |  |
|  |  |  |  |  |  |  |  | Power supply |  |  | 220V±10％ 50Hz/60Hz |  |
| Core components | Name |  | Operating system |  |  | Brand |  |  | BORUNTE |  |  |  |
|  |  |  | Casting body |  |  |  |  |  | BORUNTE |  |  |  |
|  |  |  | Servo motor |  |  |  |  |  | BORUNTE |  |  |  |
|  |  |  | Reducer |  |  |  |  |  | BORUNTE |  |  |  |
|  |  |  | Electrical component |  |  |  |  |  | BORUNTE |  |  |  |
| Oil content | J1 |  | 900ML | J2 |  |  | 800ML |  |  | J3 |  | 400ML |
|  | J4 |  | 50ML | J5 |  |  | / |  |  | J6 |  | / |
| Vulnerable parts | J4 timing belt |  | MBT233080303 Timing belt 3M-08 bandwidth - 89 teeth |  |  |  |  |  |  |  |  |  |
|  | J5 timing belt |  | MBT233080301 Timing belt 3M-08 bandwidth - 117 teeth |  |  |  |  |  |  |  |  |  |
| Robot accompany materials | USB |  |  |  |  |  |  |  |  |  |  |  |

The rated power, specifications, external dimensions, etc. of this product are subject to modification without prior notice.

Technical data and illustrations are for supply reference only and reserve the right to make changes.