Technical Recommendation

Model：BRTIRUS2550A

Name：Six axis robot

Robot version :21 version

（Without further notice if specification and exterior have been changed because of improvement. Thanks for your understanding.）

- BRTIRUS2550A robot profile

  - Introduction of BRTIRUS2550A

BRTIRUS2550A is a six axis robot developed by BORUNTE for complex applications with multiple degrees of freedom. It has an ultra long arm span of 2563mm and a maximum load of 50kg. With six degrees of flexibility, it can handle scenarios such as loading and unloading, assembly, and injection molding with ease.

High accuracy：repeated positioning accuracy: ±0.1mm

## 1.2. Application cases of BRTIRUS2550A

(1). injection molding (2). stamping (3). palletizing (4). loading and unloading, etc.

# 3. BRTIRUS2550A robot specifications

| Model |  |  | BRTIRUS2550A |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Axes |  |  | 6 axis |  |  |  |  |  |  |  |  |  |  |  |  |
| Loading ability |  |  | 50kg |  |  |  |  |  |  |  |  |  |  |  |  |
| Repeated positioning accuracy |  |  | ±0.1mm |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum arm span |  |  | 2563mm |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight |  |  | 725kg |  |  |  |  |  |  |  |  |  |  |  |  |
| Max speed | J1 |  | 84°/s |  |  |  | J2 |  |  |  |  |  | 52°/s |  |  |
|  | J3 |  | 52°/s |  |  |  | J4 |  |  |  |  |  | 245°/s |  |  |
|  | J5 |  | 223°/s |  |  |  | J6 |  |  |  |  |  | 223°/s |  |  |
| Motion range | J1 |  | ±160° |  |  |  | J2 |  |  |  |  |  | ±70° |  |  |
|  | J3 |  | -75°/+115° |  |  |  | J4 |  |  |  |  |  | ±180° |  |  |
|  | J5 |  | ±125° |  |  |  | J6 |  |  |  |  |  | ±360° |  |  |
| Allowable torque |  |  | J4 | 153N*m |  |  | J5 |  | 153N*m |  |  |  | J6 | 100N*m |  |
| Allowable moment of inertia |  |  | J4 | 28kg*m2 |  |  | J5 |  | 28kg*m2 |  |  |  | J6 | 11kg*m2 |  |
| Mounting environment | Operating temperature： 0℃~40℃ Best temperature：15℃~25℃ Relative humidity：20~80%RH（No condensation） |  |  |  |  |  |  |  | Mpa |  | 0.5~0.7MPa |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Power supply |  | 380V±10％ 50Hz/60Hz |  |  |  |  |
| Core component | Name | Operating system |  |  |  | Brand |  |  |  |  |  | BORUNTE |  |  |  |
|  |  | Casting body |  |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |
|  |  | Servo motor |  |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |
|  |  | Reducer |  |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |
|  |  | Electrical component |  |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |
| Oil content | J1 | 2000ML |  |  | J2 |  |  | 2000ML |  | J3 |  |  |  |  | 700ML |
|  | J4 | 600ML |  |  | J5 |  |  | 200ML |  | J6 |  |  |  |  | 200ML |
| Vulnerable parts | J5 timing belt |  | MBT233150515 timing belt 5M-15 Wide:130 teeth black HP |  |  |  |  |  |  |  |  |  |  |  |  |
| Robot accompanying materials |  |  | USB |  |  |  |  |  |  |  |  |  |  |  |  |

The rated power, specifications, external dimensions, etc. of this product are subject to modification without prior notice.

Technical data and illustrations are for supply reference only and reserve the right to make changes.