Technical Recommendation

Model：BRTIRUS1820A

Name：Six axis robot

Robot version :21 version

（Without further notice if specification and exterior have been changed because of improvement. Thanks for your understanding.）

- BRTIRUS1820A robot profile

  - Introduction of BRTIRUS1820A

BRTIRUS1820A is a six axis robot developed by BORUNTE for complex applications with multiple degrees of freedom. 20kg loading ability, 1895mm arm span. Adopting a lightweight arm design, the mechanical structure is compact and concise, and can perform flexible operations in a narrow workspace under high-speed motion, meeting the needs of flexible production. With six degrees of flexibility, it can freely handle scenarios such as loading and unloading, injection molding, die-casting, assembling, gluing, polishing, and testing. Suitable for injection molding machines: 500T-1300T.

High IP code：Wrist: IP54 Body: IP40

High accuracy：repeated positioning accuracy: ±0.05mm

## 1.2. Application cases of BRTIRUS1820A

(1). injection molding (2). stamping (3). transporting (4). loading and unloading, etc.

| Model | BRTIRUS1820A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Axes | 6 axis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Loading ability | 20kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IP code | Body: IP40 Wrist: IP54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Repeated positioning accuracy | ±0.05mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum arm span | 1895mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight | 230kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max speed | J1 | 110°/s |  |  |  |  |  |  | J2 |  | 146°/s |  |  |  |  |  |
|  | J3 | 134°/s |  |  |  |  |  |  | J4 |  | 273°/s |  |  |  |  |  |
|  | J5 | 240°/s |  |  |  |  |  |  | J6 |  | 375°/s |  |  |  |  |  |
| Motion range | J1 | ±155° |  |  |  |  |  |  | J2 |  | -140°/+65° |  |  |  |  |  |
|  | J3 | -75°/+110° |  |  |  |  |  |  | J4 |  | ±180° |  |  |  |  |  |
|  | J5 | ±115° |  |  |  |  |  |  | J6 |  | ±360° |  |  |  |  |  |
| Mounting environment | Operating temperature： 0℃~40℃ Best temperature：15℃~25℃ Relative humidity：20~80% RH（No condensation） |  |  |  |  |  |  |  |  |  | Mpa |  |  |  | 0.5~0.7 MPa |  |
|  |  |  |  |  |  |  |  |  |  |  | Power supply |  |  |  | 220V±10％ 50Hz/60Hz |  |
| Allowable torque | J4 | 52N*m |  |  |  | J5 |  | 56N*m |  |  |  | J6 |  | 51.2N*m |  |  |
| Allowable moment of inertia | J4 | 0.664048kg*m2 |  |  |  | J5 |  | 1.12kg*m2 |  |  |  | J6 |  | 0.3968kg*m2 |  |  |
| Core components | Name | Operating system |  |  |  |  | Brand |  |  |  | BORUNTE |  |  |  |  |  |
|  |  | Casting body |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |  |  |
|  |  | Servo motor |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |  |  |
|  |  | Reducer |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |  |  |
|  |  | Electrical component |  |  |  |  |  |  |  |  | BORUNTE |  |  |  |  |  |
| Oil content | J1 |  | 1400ML |  | J2 |  |  |  |  | 1000ML |  |  | J3 |  |  | 800ML |
|  | J4 |  | 250ML |  | J5 |  |  |  |  | / |  |  | J6 |  |  | / |
| Vulnerable parts | Timing belt |  |  | MBT233080505 timing belt 5M-8 bandwidth -85 tooth |  |  |  |  |  |  |  |  |  |  |  |  |
| Robot accompanying materials | USB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- BRTIRUS1820A robot specifications

The rated power, specifications, external dimensions, etc. of this product are subject to modification without prior notice.

Technical data and illustrations are for supply reference only and reserve the right to make changes.