Technical Recommendation

Model：BRTIRPZ1825A

Name：Four axis robot

Robot version:21 version

（Without further notice if specification and exterior have been changed because of improvement. Thanks for your understanding.）

# 1. BRTIRPZ1825A robot profile

  - 1.1.Introduction of BRTIRPZ1825A

BRTIRPZ1825A is a four axis robot developed by BORUNTE for certain monotonous, frequent, and repetitive long-term operations, as well as hazardous and harsh environments. It has an arm span of 1800mm and a maximum load of 25kg. With multiple degrees of flexibility, it can be used in loading and unloading, handling, unstacking, and stacking.

High accuracy: Repeated positioning accuracy: ±0.08mm.

## 1.2. Application cases of BRTIRPZ1825A

(1). stamping (2). palletizing (3). loading and unloading, etc.

# 2. BRTIRPZ1825A robot specifications

| Model | BRTIRPZ1825A |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Axes | 4 axis |  |  |  |  |  |  |
| Loading ability | 25kg |  |  |  |  |  |  |
| Repeated positioning accuracy | ±0.08mm |  |  |  |  |  |  |
| Maximum arm span | 1800mm |  |  |  |  |  |  |
| Weight | 256kg |  |  |  |  |  |  |
| Max speed | J1 | 175°/s |  |  | J2 | 135°/s |  |
|  | J3 | 123°/s |  |  | J4 | 300°/s |  |
| Motion range | J1 | ±155° |  |  | J2 | -78°/+30° |  |
|  | J3 | -62°/+ 70° |  |  | J4 | ±360° |  |
|  | R34 | 65°~170° |  |  | / | / |  |
| Allowable torque | J4 | 62.3N*m |  |  |  |  |  |
| Allowable moment of inertia | J4 | 0.145KG*m² |  |  |  |  |  |
| Mounting environment | Operating temperature： 0℃-40℃ Best temperature：15℃-25℃ Relative humidity：20-80%RH（No condensation） |  |  |  | Mpa | 0.5~0.7MPa |  |
|  |  |  |  |  | Power supply | 220V±10％ 50HZ/60Hz |  |
| Core components | Name | Operating system |  |  | Brand | BORUNTE |  |
|  |  | Casting body |  |  |  | BORUNTE |  |
|  |  | Servo motor |  |  |  | BORUNTE |  |
|  |  | Reducer |  |  |  | BORUNTE |  |
|  |  | Electrical component |  |  |  | BORUNTE |  |
| RV Oil content | J1 | 1700ML | J2 | 860ML | J3 |  | 660ML |
|  | J4 | / |  |  |  |  |  |
| Vulnerable parts | None |  |  |  |  |  |  |
| Robot accompanying materials | USB |  |  |  |  |  |  |

The rated power, specifications, external dimensions, etc. of this product are subject to modification without prior notice. Technical data and illustrations are for supply reference only and reserve the right to makechanges.